Development of the Isobaric System Equations

© John Kreskovsky

August 2005

We begin by examining the equation which governs the motion of a single driver in a sealed box. For the moment we will only consider the case of linear motion. The driver is governed by an ordinary differential equation, expressed in dimensionless form as,

F(t) = x” + Ω (1/Qms + 1/Qes) x’ + Ω2 (1 + α) x					(1)

Here F(t) is the driving force, x the driver displacement, x’ is the driver velocity, x” is the driver acceleration, α is the compliance ratio, Vas/Vbox , Qms and Qes are the mechanical and electrical Q of the driver in free air, and Ω is a dimensionless frequency,

Ω = ωn / ω										(2)

where ωn is the driver’s natural frequency and ω is the frequency of the input signal. We note that as ω gets large Ω will tends to zero. Thus, at high frequency only the acceleration term is relevant and we have what is referred to as the mass controlled region. As ω gets very small Ω2 becomes the dominant term and we are in the compliance controlled region. Around the frequency where ω = ωn, Ω is close to 1.0 and all terms in the equation are equally important. For a sinusoidal input Eq (1) gives rise to a simple harmonic output.

We next consider what happens when we have an isobaric system where a second driver is coupled to the first through a small volume of trapped air, the “isobaric” chamber. In this case we have two equations, one for each driver. The rear driver motion is governed by the equation,

F(t) = xr” + Ω (1/Qms + 1/Qes) xr’ + Ω2 (1 + α) xr  + Ω2β (xr –xf)			(3)

The front driver is governed by

F(t) = xf” + Ω (1/Qms + 1/Qes) xf’ + Ω2 xf  - Ω2β (xr –xf)				(4)

Here β is the compliance ratio: Vas divided by the volume of the isobaric chamber, Vchamber, and the last term in Eqs 2 and 4 is the term which couples the motion of the front and rear drivers. We note that Eqs 2 and 3 are identical except for the rear box compliance term and the sign of the coupling term. The sign of the coupling term is different because the pressure in the isobaric chamber exerts a force on each driver which is equal in magnitude but opposite in direction.

Now let’s examine the sources of some of the misconceptions about isobaric systems. First, the very name, isobaric, implies that the pressure in the isobaric volume is constant. This would require that either (xr –xf) equal zero or a constant since the force exerted on the drivers by the pressure in the isobaric chamber is +/- β (xr –xf). If this were the case then the drivers would have to have exactly the same motion, to within a constant,

xr (t) = xf(t) + c									(5)

While this may seem reasonable at first glance, if β (xr –xf) = a constant = C, Eqs 3 and 4 become

F(t) = xr” + Ω (1/Qms + 1/Qes) xr’ + Ω2 (1 + α) xr  + C				(6)

F(t) = xf” + Ω (1/Qms + 1/Qes) xf’ + Ω2 xf  - C					(7)

We don’t need to be rocket scientists to see that Eqs 6 and 7 are different and therefore their solutions will not yield xr (t) = xf(t) + c. Thus the assumption that the pressure in the “isobaric” chamber is constant can not be correct. However, let us continue. We can also sum Eqs 3 and 4 with the result that

2F(t) = (xr” + xf”) + Ω (1/Qms + 1/Qes) (xr’+ xf’) + Ω2 (xr + xf )  + Ω2 α xr  	(8)

Here we see that regardless of the motion of the drivers the coupling term cancels. If we again assume, incorrectly, that xr (t) = xf(t) and the front and rear driver undergo the same motion, then Eq(8) becomes, by dividing by 2,

F(t) = x” + Ω (1/Qms + 1/Qes) x’ + Ω2 (1 + α/2) x  					(9)

where

x = (xr + xf) / 2. 									(10)

We note the last term involves the compliance ratio divided by 2. Since the compliance ratio is α = Vas/ Vbox,

α/2 = Vas/2Vbox									(11)

which leads to the statement that Vas for the isobaric system is ½ Vas of the base driver, or conversely, the box looks like it has twice the volume. As noted, this requires that xr (t) = xf(t), or more generally, xr (t) = xf(t) + c.

Let us spend one additional moment examine the behavior of Eqs 3 and 4. Under the conventional reasoning the response of the isobaric system is the same as if one driver were placed in a box of twice the volume. Looking at Eq(4), the only way the front driver, the driver of relevance since it generates the sound pressure, could have this response is if the force exerted upon it by the pressure in the isobaric chamber were identical to that exerted upon if it were loaded by a sealed box having twice the volume of the rear box used in the isobaric system. Thus,

- β (xr –xf) = (α/2) xf = (Vas/2Vbox(iso)) xf = (Vas/V b(single)) xf			(12)

This is actually a more profound statement that it may appear. First it says that the pressure in the isobaric chamber can not be constant. Second, it states that (xr –xf) ≠ 0 regardless of how small is Vchamber (or how big β is). In fact, it says that

xr  = (1 - α/2β) xf  = (1 - Vchamber/ 2Vbox(iso)) xf					(13)

Eq(13) has to be taken with an understanding of it’s limits. For β large (small Vchamber), the front and rear drivers are coupled through a very stiff spring in which case the motion of the front and rear driver will approach each other and Eq 13 is indicative of the relative motion of the drivers. It also indicates that amplitude of the rear driver motion will always be less than that of the front driver. However, as β  α/2 from above (that is, starting from a value of β much greater than α/2) Eq (13) indicates that the rear driver motion will cease. This can’t happen because we know  that as β gets smaller the coupling between Eqs 3 and 4 is weaker and ultimately, as β goes to zero, each driver becomes independent of the other driver’s motion. Thus Eq (13) is at best an approximation which is valid only for large β. Eq(13) is slightly different from Eq(6) on my web page because it was derived from a different assumption. However, when α/2β is mush less then 1, 1/(1 - α/2β) ≈ (1 + α/2β) and Eq(13) and Web Eq(6) are fundamentally the same.

All the discussion so far has dealt only with linear systems and serves only to indicate that even in the linear case the isobaric system doesn’t exactly behave in an isobaric fashion. We shall next look at the actual behavior of the system with and without the inclusion of nonlinear effects. The nonlinear effects to be considered will be the nonlinearity of the air compression/expansion process in the rear box and the isobaric chamber as well as nonlinearity in the suspension compliance. Nonlinearity of the motor is not considered. To introduce the effects of nonlinear driver suspension and air compression we need only make a few observations and simple modifications to our governing equations. Both these nonlinearities affect the compliance terms. For the trapped air we assume the compression/expansion process is governed by a polytropic process,

PVk = constant									(14)

where P is the pressure, V the volume and k is the polytropic exponent. k = 1 for an isothermal process and 1.4 for an isentropic process. An isentropic process is assumed here. The variation in volume of the rear box is given as

V = Vo + Sd xr									(15)

where Vo is the box volume when the driver is at rest. Thus the pressure in the box is

P = constant/( Vo + Sd xr )k								(16)

Without going through the details Eq (16) leads to the nonlinear compliance

Cabnl = Cab(1 + Sd xr /Vo) k+1							(17)

A similar expression is obtained for the nonlinear compliance of the isobaric chamber with (xf – xr) replacing xr in Eq (17).

For the suspension compliance a simple generic relationship is used,

Cms(x) = Cms(x=0) * (tanh(SF*(1.25 Xmax - |x –xoff|)/Xmax)  + 1

.+ 0.1exp(-|x|/(5 Xmax)) ) / 2.1						(18)

Here SF is a shape factor between 2.0 and 10 which control the shape of the compliance with excursion and xoff is used to shift the curve in the + or – x direction to introduce asymmetry in Cms(x). Figure 2 show the result for SF = 2.0.

Figure 2. Suspension compliance variation with displacement for SF = 2.0.